Complete the timing diagram for the BRAM defined on the opposing page.

| Addr | writeInput | WREN=cw(4) | WE=cw(3,2) | RDEN=cw(5) | readOutput |
| --- | --- | --- | --- | --- | --- |
| 0x0 | 0x2AAA0 |  |  |  |  |
| 0x1 | 0x2AAA1 |  |  |  |  |
| 0x2 |  |  |  |  |  |
| 0x3 |  |  |  |  |  |
| 0x4 |  |  |  |  |  |
| 0x5 |  |  |  |  |  |
| 0x6 |  |  |  |  |  |
| 0x7 |  |  |  |  |  |
| 0x8 | 0x2AAA8 |  |  |  |  |
| 0x9 |  |  |  |  |  |
| 0xA |  |  |  |  |  |
| 0xB |  |  |  |  |  |
| 0xC |  |  |  |  |  |
| 0xD |  |  |  |  |  |
| 0xE | 0x2AAAE |  |  |  |  |
| 0xF |  |  |  |  |  |

-------------------------------------------------------------------------

-- Name:	Maj Jeff Falkinburg

-- Date:	Jan 29, 2017

-- File:	lec12_dp.vhdl

-------------------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.ALL;			-- contains the unsigned data type

library UNIMACRO;				-- This contains links to the Xilinx block RAM

use UNIMACRO.vcomponents.all;

entity lec12Dual_dp is

Port(	clk: in  STD_LOGIC;

n_reset : in  STD_LOGIC;

cw: std_logic_vector(5 downto 0));

end lec12Dual_dp;

architecture behavior of lec12Dual_dp is

signal writeInput, readOutput: std_logic_vector(17 downto 0);

signal addrWrite, addrRead: unsigned(9 downto 0);

signal vecAddrWrite, vecAddrRead : std_logic_vector(9 downto 0);

signal reset: std_logic;

begin

process(clk)

begin

if (rising_edge(clk)) then

if (n_reset = '0') then

addrWrite <= (others => '0');

elsif (cw(1 downto 0) = "01") then

addrWrite <= addrWrite + 1;

elsif (cw(1 downto 0) = "11") then

addrWrite <= (others => '0');

end if;

end if;

end process;

addrRead <= addrWrite - 1;				-- Have the read follow the writes

writeInput <= "10101010101010" & vecAddrWrite(3 downto 0);

reset <= not n_reset;				-- BRAM reset is active high

vecAddrWrite <= std_logic_vector(addrWrite);	-- type conversion

vecAddrRead <= std_logic_vector(addrRead);

sampleMemory: BRAM_SDP_MACRO

generic map (

BRAM_SIZE => "18Kb", 	-- Target BRAM, "18Kb" or "36Kb"

DEVICE => "7SERIES", -- Target device: "VIRTEX5", "VIRTEX6", "SPARTAN6, 7SERIES"

DO_REG => 0, 			-- Optional output register disabled

INIT => X"000000000000000000",	-- Initial values on output port

INIT_FILE => "NONE",	-- Not sure how to initialize the RAM from a file

WRITE_WIDTH => 18, 	-- Valid values are 1-72 (37-72 valid when BRAM_SIZE="36Kb")

READ_WIDTH => 18, 	-- Valid values are 1-72 (37-72 valid when BRAM_SIZE="36Kb")

SIM_COLLISION_CHECK => "NONE", 	-- Simulation collision check

SRVAL => X"000000000000000000")	-- Set/Reset value for port output

port map (

DO => readOutput,			-- Output read data port width=READ_WIDTH

RDADDR => vecAddrRead,		-- Read address, width=FNC(BRAM_SIZE)

RDCLK => clk,	 			-- 1-bit input clock

RST => reset,			-- active high reset

RDEN => cw(5),			-- read enable

REGCE => '1',				-- read output register enable - ignored

DI => writeInput,			-- Input data port, width=WRITE_WIDTH

WE => cw(3 downto 2),		-- RAM is byte read, high or low byte

WRADDR => vecAddrWrite,		-- Write address, width=F(BRAM_SIZE)

WRCLK => clk,				-- 1-bit input write clock

WREN => cw(4));			-- 1-bit master write enable

end behavior;