-------------------------------------------------------------------------

-- File:	lec19.vhdl

-- Ctrl:	00=Hold	01=Increment		10=Load	11=reset

-------------------------------------------------------------------------

entity lec11 is

generic( N		: integer := 4);

port(	clk		: in  STD_LOGIC;

reset_n 		: in  STD_LOGIC;

ctrl		: in std_logic_vector(1 downto 0);

D		: in unsigned (N-1 downto 0);

Q		: out unsigned (N-1 downto 0));

end lec11;

architecture behavior of lec11 is

signal processQ: __________________________________ ;

begin

process(clk)

begin

if (rising_edge(clk)) then

if (reset_n = '0') then

processQ <= ___________________ ;

elsif (ctrl = "01") then

processQ <= ___________________ ;

elsif (ctrl = "10") then

processQ <= ___________________ ;

elsif (crtl = "11") then

processQ <= ___________________ ;

end if;

end if;

end process;

Q <= processQ;

end behavior;

------------------------------------------------------------------------------

-- File:	my_counter_ip_v1_0_S00_AXI.vhd

------------------------------------------------------------------------------

use ieee.numeric_std.all;

entity my_counter_ip_v1_0_S00_AXI is

generic (

-- Width of S_AXI data bus

C_S_AXI_DATA_WIDTH	: integer	:= 32;

-- Width of S_AXI address bus

C_S_AXI_ADDR_WIDTH	: integer	:= 7

);

port (

-- Users to add ports here

LED	  : out std_logic_vector(7 downto 0);

-- User ports ends

-- Do not modify the ports beyond this line

-- Global Clock Signal

S_AXI_ACLK	: in std_logic;

-- Global Reset Signal. This Signal is Active LOW

S_AXI_ARESETN	: in std_logic;

… lots of other stuff …);

architecture arch_imp of my_counter_ip_v1_0_S00_AXI is

------------------------------------------------

---- Signals for user logic register space example

--------------------------------------------------

component lec11 is

generic (N: integer := 4);

Port(    clk: in  STD_LOGIC;

reset_n : in  STD_LOGIC;

ctrl: in std_logic_vector(1 downto 0);

D: in unsigned (N-1 downto 0);

Q: out unsigned (N-1 downto 0));

end component;

signal _________ : unsigned (7 downto 0);

begin

-- Address decoding for reading registers

loc_addr := axi_araddr(ADDR_LSB + OPT_MEM_ADDR_BITS downto ADDR_LSB);

case loc_addr is

when b"00000" =>

reg_data_out <= X"000000" & std_logic_vector(Q);

when b"00001" =>

reg_data_out <= slv_reg1;

when b"00010" =>

reg_data_out <= slv_reg2;

… lots more stuff here

end case;

-- Add user logic here

counter: ___________

generic map (____)

port map(	clk =>		___________________________,

reset_n =>	___________________________,

ctrl =>		slv_reg1(1 downto 0),

D =>		unsigned(slv_reg0(7 downto 0)),

Q =>		______________________________);

LED <= std_logic_vector(Q);

-- User logic ends

------------------------------------------------------------------------------

-- my_counter_ip_v1_0.vhd

------------------------------------------------------------------------------

use ieee.numeric_std.all;

entity my_counter_ip_v1_0 is

generic (

C_S00_AXI_DATA_WIDTH	: integer	:= 32;

C_S00_AXI_ADDR_WIDTH	: integer	:= 7

);

port (

-- Users to add ports here

LED : out std_logic_vector(7 downto 0);

-- User ports ends

-- Do not modify the ports beyond this line

-- Ports of Axi Slave Bus Interface S00_AXI

s00_axi_aclk	: in std_logic;

s00_axi_aresetn	: in std_logic;

… lots of other stuff …);

architecture arch_imp of my_counter_ip_v1_0 is

-- component declaration

component my_counter_ip_v1_0_S00_AXI is

generic (

C_S_AXI_DATA_WIDTH	: integer	:= 32;

C_S_AXI_ADDR_WIDTH	: integer	:= 7

);

port (

LED            : out std_logic_vector(7 downto 0);

S_AXI_ACLK	: in std_logic;-- Instantiation of Axi Bus Interface S00_AXI

… lots of other stuff …);

my_counter_ip_v1_0_S00_AXI_inst : my_counter_ip_v1_0_S00_AXI

port map(	S_AXI_ACLK	=> S_AXI_ACLK,

S_AXI_ARESETN	=> S_AXI_ARESETN,

S_AXI_WDATA	=> S_AXI_WDATA,

… lots of other stuff …);my_counter_ip_v1_0_S00_AXI_inst : my_counter_ip_v1_0_S00_AXI

generic map (

C_S_AXI_DATA_WIDTH	=> C_S00_AXI_DATA_WIDTH,

C_S_AXI_ADDR_WIDTH	=> C_S00_AXI_ADDR_WIDTH )

port map (

LED => LED,

S_AXI_ACLK	=> s00_axi_aclk,

S_AXI_ARESETN	=> s00_axi_aresetn,

… lots of other stuff …);

/*--------------------------------------------------------------------

-- Name:	Prof Jeff Falkinburg

-- Date:	Feb 16, 2017

-- File:	lec19.c

-- Event:	Lecture 19

-- Crs:	CSCE 436

--

-- Purp:	MicroBlaze Tutorial that implements a custom IP to microBlaze.

--

-- Documentation:	MicroBlaze Tutorial

--

-- Academic Integrity Statement: I certify that, while others may have

-- assisted me in brain storming, debugging and validating this program,

-- the program itself is my own work. I understand that submitting code

-- which is the work of other individuals is a violation of the honor

-- code.  I also understand that if I knowingly give my original work to

-- another individual is also a violation of the honor code.

-------------------------------------------------------------------------*/

/***************************** Include Files ********************************/

#include "xparameters.h"

#include "stdio.h"

#include "xstatus.h"

#include "platform.h"

#include "xil_printf.h"				// Contains xil_printf

#include <xuartlite_l.h>				// Contains XUartLite_RecvByte

#include <xil_io.h>					// Contains Xil_Out8 and its variations

/************************** Constant Definitions ****************************/

/*

* The following constants define the slave registers used for our Counter PCORE

*/

#define countQReg		0x44a00000		// 8 LSBs of slv_reg0 read=Q, write=D

#define countCtrlReg	0x44a00004		// 2 LSBs of slv_reg1 are control

#define countRollReg	0x44a00008		// 1 LSBs of slv_reg2 for roll

/*

* The following constants define the Counter commands

*/

#define count_HOLD		0x00		// The control bits are defined in the VHDL

#define count_COUNT		0x01		// code contained in lec18.vhdl.  They are

#define count_LOAD		0x02		// added here to centralize the bit values in

#define count_RESET		0x03		// a single place.

#define printf xil_printf			/* A smaller footprint printf */

#define uartRegAddr		0x40600000		// read <= RX, write => TX

/************************** Function Prototypes ****************************/

/************************** Variable Definitions **************************/

/*

* The following are declared globally so they are zeroed and so they are

* easily accessible from a debugger

*/

int main()

{

unsigned char c;

init_platform();

print("Welcome to Lecture 19\n\r");

while(1) {

c=XUartLite_RecvByte(uartRegAddr);

switch(c) {

/*-------------------------------------------------

* Reply with the help menu

*-------------------------------------------------

*/

case '?':

printf("--------------------------\r\n");

printf("	count Q = %x\r\n",Xil_In16(countQReg));

printf("--------------------------\r\n");

printf("?: help menu\r\n");

printf("o: k\r\n");

printf("c: COUNTER	count up LEDs (by x26)\r\n");

printf("s: COUNTER	start counter\r\n");

printf("l: COUNTER	load counter\r\n");

printf("r: COUNTER	reset counter\r\n");

printf("f: flush terminal\r\n");

break;

/*-------------------------------------------------

* Basic I/O loopback

*-------------------------------------------------

*/

case 'o':

printf("k \r\n");

break;

/*-------------------------------------------------

* Tell the counter to count up

*-------------------------------------------------

*/

case 'c':

Xil_Out8(countCtrlReg,count_COUNT);

Xil_Out8(countCtrlReg,count_HOLD);

break;

/*-------------------------------------------------

* Start the counter to count up

*-------------------------------------------------

*/

case 's':

Xil_Out8(countCtrlReg,count_COUNT);

break;

/*-------------------------------------------------

* Stop the counter from counting

*-------------------------------------------------

*/

case 'S':

Xil_Out8(countCtrlReg,count_HOLD);

break;

/*-------------------------------------------------

* Tell the counter to load a value

*-------------------------------------------------

*/

case 'l':

printf("Enter a 0-9 value to store in the counter: ");

c=XUartLite_RecvByte(uartRegAddr) - 0x30;

Xil_Out8(countQReg,c);				// put value into slv_reg1

Xil_Out8(countCtrlReg,count_LOAD);		// load command

printf("%c\r\n",c+0x30);

break;

/*-------------------------------------------------

* Reset the counter

*-------------------------------------------------

*/

case 'r':

Xil_Out8(countCtrlReg,count_RESET);		// reset command

break;

/*-------------------------------------------------

* Clear the terminal window

*-------------------------------------------------

*/

case 'f':

for (c=0; c<40; c++) printf("\r\n");

break;

/*-------------------------------------------------

* Unknown character was

*-------------------------------------------------

*/

default:

printf("unrecognized character: %c\r\n",c);

break;

} // end case

}

cleanup_platform();

return 0;

} // end main