Direct Digital Synthesis (DDS) is a technique to create periodic waveforms with very precise frequency control using a system with a fixed clock frequency.  The periodic function is stored in a look-up table, like the one below, which represents one cycle of a sine wave.

int8 sin[64] = {128,141,153,165,177,189,200,210,219,227,235,241,246,250,253,255,

255,254,252,248,244,238,231,223,214,205,194,183,171,159,147,134,

122,109, 97, 85, 73, 62, 51, 42, 33, 25, 18, 12,  8,  4,  2,  1,

1,  3,  6, 10, 15, 21, 29, 37, 46, 56, 67, 79, 91,103,115,128};

You are:

1) Given a lookup table with 2^N values corresponding to one wavelength of a function

2) Given a sampling rate or a play back rate of f updates/second

3) Given a phase increment x, which every 1/f is added to the index of the LUT

f updates    x values     1 cycle     f*x

--------- * ---------  * --------- =  --- hz

1 second      update     2^N values   2^N

Questions:

Assuming an update rate of 48kHz, a LUT with 1024 entries, and a phase increment

of x, expressed as a 10.6 fixed point number.  Answer the following questions.

- What is the maximum frequency we could generate?

- What is the minimum frequency we can generate?

- What is the smallest change in frequency we can make with the phase increment?

- What phase increment generates a frequency of 440hz?

- How did I arrive at the format of the phase increment?