Use the sinusoidal lookup table at left to generate a sine wave using a Q4.4 format phase increment of 0001.1011.  Plot the “Base” and “Base + Offset*Delta” columns in the graph below.

| 0 | 127 |
| --- | --- |
| 1 | 175 |
| 2 | 216 |
| 3 | 244 |
| 4 | 253 |
| 5 | 244 |
| 6 | 216 |
| 7 | 175 |
| 8 | 127 |
| 9 | 78 |
| 10 | 37 |
| 11 | 9 |
| 12 | 0 |
| 13 | 9 |
| 14 | 36 |
| 15 | 78 |

| Time | Index | Base | Delta | Offset*Delta | Base + Offset*Delta |
| --- | --- | --- | --- | --- | --- |
| 0 | 0000.0000 | 127 | 175-127 = 48 = 110000 | 0.0000*110000 = 00000.0000 | 127+0 = 127 |
| 1 | 0001.1011 | 175 | 216-175 = 41 = 101001 | 0.1011*101001 = 11100.0011 | 175+28 = 203 |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |

-------------------------------------------------------------------------------------

--	BRAM_SDP_MACRO:	Simple Dual Port RAM 7 Series

--	Source:		Xilinx HDL Libraries Guide, version 2012.4

--	Link:	https://www.xilinx.com/support/documentation/sw_manuals/xilinx2012_2/ug953-vivado-7series-libraries.pdf

--	Page:			10

-------------------------------------------------------------------------------------

sampleMemory: BRAM_SDP_MACRO

generic map (

BRAM_SIZE => "18Kb",			-- Target BRAM, "18Kb" or "36Kb"

DEVICE => "7SERIES",			-- Target device: "VIRTEX5", "VIRTEX6", "SPARTAN6", "7SERIES"

DO_REG => 0, 				-- Optional output register disabled

INIT => X"000000000000000000",	-- Initial values on output port

INIT_FILE => "NONE",			--

WRITE_WIDTH => 16,			-- Valid values are 1-72 (37-72 only valid when BRAM_SIZE="36Kb")

READ_WIDTH => 16,			-- Valid values are 1-72 (37-72 only valid when BRAM_SIZE="36Kb")

SIM_COLLISION_CHECK => "NONE", 	-- Simulation collision check

SRVAL => X"000000000000000000",	-- Set/Reset value for port output

INIT_00 => X"8BC28AF98A31896988A087D8870F8647857E84B583EC8323825A819180C88000",

INIT_01 => X"9830976A96A595DF95199452938C92C591FE913790708FA98EE18E198D528C8A",

INIT_02 => X"A462A3A2A2E0A21FA15DA09B9FD89F169E529D8F9CCB9C079B439A7F99BA98F5",

INIT_3E => X"7247717F70B76FF06F296E616D9A6CD46C0D6B476A8069BA68F5682F676A66A5",

INIT_3F => X"7ECF7E067D3D7C747BAC7AE37A1A7951788977C076F7762F7567749F73D6730E")

port map (

DO => DO,					-- Output read data port, width defined by READ_WIDTH parameter

RDADDR => vecAddrRead,		-- Input address, width defined by port depth

RDCLK => clk,	 			-- 1-bit input clock

RST => reset,				-- active high reset

RDEN => '1',				-- read enable

REGCE => '1',				-- 1-bit input read output register enable - ignored

DI => DI,					-- Dummy write data - never used in this application

WE => "00",				-- write to neither byte

WRADDR => "0000000000",		-- Dummy place holder address

WRCLK => clk,				-- 1-bit input write clock

WREN => '0');				-- we are not writing to this RAM