Use the excel file accompanying Lesson 26 to generate the data in the table below – make sure to use the “second order” tab.  When complete, plot the data in the empty graph at right to build the phase and magnitude plots.

| Frequency Cell B2 | Gain Cell P10 | Phase Cell P9 |
| --- | --- | --- |
| 100 |  |  |
| 300 |  |  |
| 500 |  |  |
| 1,000 |  |  |
| 1,300 |  |  |
| 1,500 |  |  |
| 3,000 |  |  |
| 5,000 |  |  |
| 7,000 |  |  |
| 10,000 |  |  |
| 13,000 |  |  |
| 15,000 |  |  |

Calculate the slope of the Gain (to the right of the cut-off

frequency).   Show your work here:

Use the equations presented in class to generate the coefficients

for a 2nd order low pass filter with a cut-off frequency of 300Hz.

| Coefficient | Decimal | Fixed Point |
| --- | --- | --- |
| X[n-2] |  |  |
| X[n-1] |  |  |
| X[n-0] |  |  |
| Y[n-1] |  |  |
| Y[n-2] |  |  |

Build a project around the VHDL code for this lesson.  Plug the coefficients for your filter above into the right channel filter.  Synthesize your design, download and listen to the difference between the left and right audio channels.

-------------------------------------------------------------------------------------

Audio_Codec : Audio_Codec_Wrapper

Port map ( clk => clk,

reset_n => reset_n,

ac_mclk => ac_mclk,

ac_adc_sdata => ac_adc_sdata,

ac_dac_sdata => ac_dac_sdata,

ac_bclk => ac_bclk,

ac_lrclk => ac_lrclk,

ready => ready,

L_bus_in => LdacValue, -- left channel input to DAC

R_bus_in => RdacValue, -- right channel input to DAC

L_bus_out => LadcValue, -- left channel output from ADC

R_bus_out => RadcValue, -- right channel output from ADC

scl => scl,

sda => sda);

left_filter_lpf1000: entity work.IIR_Biquad(arch)

-- low pass  2nd order butterworth  fl = 1000Hz, Fs = 48000Hz

-- http://www.earlevel.com/main/2013/10/13/biquad-calculator-v2/

generic map(Coef_b0 => B"00_00_0000_0100_0000_0010_1001_0110_1101", 		-- +0.003916127

Coef_b1 => B"00_00_0000_1000_0000_0101_0010_1101_1010",		-- +0.007832253

Coef_b2 => B"00_00_0000_0100_0000_0010_1001_0110_1101",		-- +0.003916127

Coef_a1 => B"10_00_1011_1101_0001_0111_0011_1010_0010",		-- -1.815341083

Coef_a2 => B"00_11_0101_0010_1111_0011_0010_0001_0001")		-- +0.831005589

port map (	clk => clk,

n_reset => reset,

sample_trig => ready,

X_in => LadcValue,

filter_done => OPEN,

Y_out => L_filter_lpf1000);

process (clk)

begin

if (rising_edge(clk)) then

if reset = '0' then

LdacValue <= (others => '0');		RdacValue <= (others => '0');

elsif(ready = '1') then

if (filter_switch = "00") then

LdacValue <= LadcValue;		RdacValue <= RadcValue;

else

LdacValue <= L_filter_lpf1000;	RdacValue <= R_filter_lpf1000;

end if;

end if;

end if;

end process;